4×4 Register File with Read/Write/Clear

Bill of Materials — PCB Component List

Harshvardhan Rathod & Alok Deep Verma  |  FY B.Tech E&CE  |  2025-26

Prepared for mentor review — component sourcing from DigiKey

This document lists every component required to build the 4×4 Register File project as a PCB. All parts are sourced from DigiKey and are standard through-hole TTL components suitable for hand-soldering. Prices are approximate unit prices at single-quantity orders; bulk pricing reduces costs further. DigiKey links are clickable in the digital version of this document.

1. Integrated Circuits

All ICs are standard 74-series TTL in DIP packages. The NE555P is added as the clock source (astable mode, frequency set by external R-C network). The 74LS08 replaces the plain 7408 from the original design for better noise margin — functionally identical.

| Part No. | Description | Qty | Unit Price | DigiKey Link |
| --- | --- | --- | --- | --- |
| NE555P | 555 Timer, DIP-8 | 1 | $0.51 | digikey.com/NE555P |
| SN74LS175N | Quad D Flip-Flop, DIP-16 | 4 | $0.92 | digikey.com/SN74LS175N |
| SN74LS244N | Octal 3-State Buffer, DIP-20 | 2 | $0.90 | digikey.com/SN74LS244N |
| SN74LS139AN | Dual 2-to-4 Decoder, DIP-16 | 1 | $0.85 | digikey.com/SN74LS139AN |
| SN7402N | Quad 2-Input NOR Gate, DIP-14 | 1 | $0.70 | digikey.com/SN7402N |
| SN74LS08N | Quad 2-Input AND Gate, DIP-14 | 1 | $0.70 | digikey.com/SN74LS08N |
| SN7432N | Quad 2-Input OR Gate, DIP-14 | 1 | $0.70 | digikey.com/SN7432N |

Note: 74LS-series devices are recommended over plain 74-series for lower power and better noise immunity. All are pin-compatible replacements.

2. Tactile Switches

Three identical tactile switches are used — one each for Clear, Write Enable (RW), and Clock. The default logic state is set by pull-up or pull-down resistors on the PCB rather than by the switch itself, so all three use the same SPST-NO part.

| Part No. | Description | Qty | Unit Price | DigiKey Link |
| --- | --- | --- | --- | --- |
| PTS645VK83-2 LFS | Tactile Switch SPST-NO, through-hole, 6mm | 3 | $0.34 | digikey.com/PTS645VK83 |

| Button | Default State | PCB Resistor Required |
| --- | --- | --- |
| Clear | HIGH (inactive) → pressed = LOW | 10 kΩ pull-UP to VCC |
| Write Enable | LOW (read mode) → pressed = HIGH | 10 kΩ pull-DOWN to GND |
| Clock | LOW → pressed = HIGH (one pulse) | 10 kΩ pull-DOWN to GND |

3. LEDs

20 LEDs total. 4 LEDs form the output bus display (one per bit, driven by the 74LS244 read bus through 220 Ω resistors). 16 LEDs form the always-on 4×4 monitor matrix showing the live contents of all four registers simultaneously — these connect directly to the Q outputs of each 74175 through individual 220 Ω resistors.

| Part No. | Description | Qty | Unit Price | DigiKey Link |
| --- | --- | --- | --- | --- |
| WP7113ID | 5mm Red Diffused LED, T-1 3/4, through-hole (output bus) | 4 | $0.25 | digikey.com/WP7113ID |
| WP7113ID | 5mm Red Diffused LED, T-1 3/4, through-hole (monitor matrix) | 16 | $0.25 | digikey.com/WP7113ID |

Tip: Use a different LED colour (e.g. WP7113GD green) for the 4 output bus LEDs to visually distinguish them from the 16 monitor matrix LEDs. Both are available at the same price on DigiKey.

4. Passive Components

Resistors and capacitors. The 220 Ω resistors limit LED current to approximately 6.4 mA per LED (safe for 74LS244 source current of 15 mA max). The 10 kΩ resistors set default button states. The 0.1 µF ceramic capacitors are placed one per IC as close to the VCC pin as possible to suppress switching noise.

| Part No. | Description | Qty | Unit Price | DigiKey Link |
| --- | --- | --- | --- | --- |
| CFR-25JB-52-220R | 220 Ω ¼W Carbon Film Resistor, axial (LED current limiting) | 20 | $0.10 | digikey.com/CFR-25JB-52-220R |
| CFR-25JB-52-10K | 10 kΩ ¼W Carbon Film Resistor, axial (button pull-up/down) | 5 | $0.10 | digikey.com — search CFR-25JB-52-10K |
| C320C104J5R5TA7301 | 0.1 µF 50V Ceramic Capacitor, radial (power decoupling) | 10 | $0.30 | digikey.com/C320C104J5R5TA7301 |

5. Total Cost Estimate

| Category | Estimated Cost |
| --- | --- |
| ICs — all 11 chips | ~$9.50 |
| Tactile switches (×3) | ~$1.00 |
| LEDs 5mm red (×20) | ~$5.00 |
| 220 Ω resistors (×20) | ~$2.00 |
| 10 kΩ resistors (×5) | ~$0.50 |
| 0.1 µF capacitors (×10) | ~$3.00 |
| PCB fabrication (5 pcs) | ~$6.00 |
| TOTAL (single board) | ~$27.50 |

All prices are single-quantity estimates from DigiKey. Ordering in packs of 10–25 resistors/capacitors reduces per-unit cost significantly.

PCB fabrication: JLCPCB or PCBWay — 5 boards of this size (~100×80mm) typically cost $5–8 including shipping.

India alternative: Mouser India, Robu.in, and Evelta.com stock most of these parts locally and avoid international shipping charges.

6. 555 Timer Clock Network

The NE555P in astable mode requires two resistors and one capacitor to set the clock frequency. These are not listed in the main BOM as their values depend on the desired frequency. Recommended values for a 1 Hz visible clock (useful for demos):

| Component | Value | DigiKey Part (example) | Purpose |
| --- | --- | --- | --- |
| R1 | 10 kΩ | CFR-25JB-52-10K | Charge path |
| R2 | 68 kΩ | CFR-25JB-52-68K | Charge + discharge path |
| C (timing) | 10 µF electrolytic | UVR1H100MDD | Sets frequency |
| C (bypass) | 0.01 µF ceramic | Already in passives list | Pin 5 bypass |

Formula: f ≈ 1.44 / ((R1 + 2×R2) × C). With R1=10kΩ, R2=68kΩ, C=10µF → f ≈ 1 Hz. Replace C with 100nF for ~1kHz fast clock.

Document prepared with AI assistance (Claude, Anthropic) | March 2026